# Track A: What Needs To Be Done

41014 Sensors and Control in Mechatronics Systems, Assessment 4 Integrated Project (40%). Track A: TurtleBot3 visual waypoint navigation and ArUco docking in MuJoCo.

# 1. The task in one paragraph

Drive a TurtleBot3 (2D LiDAR + front camera) through a room with obstacles, using the LiDAR to avoid them, and visually dock at a charging station marked with an ArUco tag. Three things are compulsory: a state-space model of the robot, an EKF that fuses wheel-encoder odometry with range/bearing (or pose) measurements from the camera, and a state-feedback or LQR trajectory tracker. Hard rule: the controller must only ever use the EKF estimate, never simulator ground truth. Using ground truth in the loop is an automatic Fail.

Marking: group mark G = Parts A–E (20 each, 75% weight) from the GitHub page, code and video; individual viva V (25% weight). Final = 0.75G + 0.25V. Simulation must be shown working by Week 12 if the team wants the optional hardware induction (hardware is only needed for High Distinction).

# 2. Where the project is now

The folder already contains a MuJoCo starter (turtlebot_sim.py + turtlebot3/scene.xml). What exists and what is missing:

| Already there | Missing (our work) |
| --- | --- |
| Differential-drive TurtleBot3 “Burger-scale” model, wheel radius 0.033 m, track 0.160 m, 500 Hz physics | ArUco-marked charging station in the scene |
| 4 m × 4 m room, one box and one cylinder obstacle | Sensor noise, encoder quantisation, delays, camera calibration |
| Wheel encoders (pos + vel), 72-beam LiDAR (5° steps, clipped at 3.5 m) | Marker detection and range/bearing or pose measurement |
| front_camera (fovy 60°) usable with mujoco.Renderer | State-space model + controllability/observability analysis |
| Manual WASD keyboard driving; v/ω → wheel-speed conversion | EKF, path planner, LQR tracker, docking controller, state machine |
| verification.json: basic load/motion/LiDAR/render checks pass (MuJoCo 3.15.0) | Evaluation scripts, trials, metrics, plots, GitHub page, video |

Clean-up: the UR3e files (universal_robots_ur3e/, MJMODEL.TXT, ur3_script_backups/, move_ur3e.py, simulate_ur3e.py) belong to Track B and should be removed from the Track A repository. The starter was generated with Codex, so that must go in the Generative AI declaration.

# 3. Work required, by marking criterion

Each criterion is 20 marks. “Pass” = it works in the nominal case. Credit needs our own derivations and defended parameters with numbers. Distinction needs repeated trials, failure diagnosis and quantified trade-offs. HD needs something beyond the brief (or real hardware).

## Part A — Modelling (state-space, controllability, observability)

- Derive the unicycle model for this platform: state x = [x, y, θ]ᵀ, input u = [v, ω]ᵀ, with v = r(ωR + ωL)/2 and ω = r(ωR − ωL)/L, r = 0.033 m, L = 0.160 m.

- Discretise it (Δt of the control loop) for use in both the EKF and controller.

- Linearise about a reference trajectory to get the tracking-error model ė = A(t)e + B(t)ũ (A, B from the Jacobians).

- Controllability: show rank of [B AB A²B] = 3 when reference v ≠ 0, and explain why it is lost when the robot is stationary (non-holonomic constraint, Brockett). This motivates the separate docking strategy.

- Observability: odometry alone cannot observe absolute pose (drift). Show that with a range/bearing measurement to a marker of known position the pose becomes observable, and which states are lost when the marker leaves the camera view.

- Optional (Credit+): add a first-order wheel-motor lag identified from a step response in the sim; state all assumptions (no slip, flat floor, rigid body).

## Part B — Sensing (calibration, noise, frames)

- Add the charging station to scene.xml: a dock body with an ArUco tag as a textured plane (generate the PNG with OpenCV cv2.aruco, e.g. DICT_4X4_50, ~0.10 m side).

- Camera calibration: compute intrinsics analytically from fovy and render resolution (fy = H / (2·tan(fovy/2)), cx = W/2, cy = H/2), then verify with a checkerboard calibration in sim (cv2.calibrateCamera) and report the difference.

- Marker measurement: cv2.aruco detection + solvePnP → marker pose in camera frame → range and bearing to the robot.

- Noise: add realistic noise on purpose and document values — encoder quantisation (e.g. 4096 ticks/rev like the real XL430 motors), LiDAR range noise (~1–2 cm), pixel noise / motion blur on the camera, optional latency.

- Measure noise statistics from logged data (variance of range/bearing vs distance and angle) — these become R in the EKF.

- Frames: world/map, odom, base_link, laser, camera (MuJoCo camera looks along −Z; OpenCV optical frame is +Z forward, +Y down — the conversion must be explicit), marker. Draw a frame diagram for the project page.

- LiDAR processing: convert scans to points in world frame using the estimated pose, build an occupancy grid or obstacle list.

## Part C — Estimation (EKF)

- Prediction step from encoder odometry using the discrete model; process noise Q derived from encoder noise/slip (propagate wheel noise through the Jacobian w.r.t. inputs).

- Update step with marker range/bearing (or full pose from solvePnP) given the known marker pose in the map; R from Part B statistics.

- Handle marker-out-of-view: prediction only, covariance grows — show this in plots.

- Outlier rejection with a Mahalanobis/χ² gate on the innovation.

- Report performance: position/heading RMSE vs ground truth, 3σ bounds, NEES and NIS consistency tests.

- Distinction-level: tuning study (e.g. R ×0.1, ×1, ×10) showing consistency vs responsiveness trade-off. HD option: compare against an LiDAR-scan-matching update, a UKF, or add an IMU.

## Part D — Control (planner + LQR tracker + docking)

- Planner: A* (or RRT) on the LiDAR occupancy grid from start to a pre-dock waypoint in front of the marker; smooth into a time-parametrised reference (x_r, y_r, θ_r, v_r, ω_r).

- LQR tracker on the linearised error model (discrete, gain-scheduled on v_r or recomputed each step). Choose Q and R weights deliberately and justify them (Bryson’s rule as a starting point).

- Respect actuator limits: wheel command clipped at 8 rad/s (≈0.26 m/s); show control effort plots.

- Docking phase: once marker is in view, switch to a docking controller (LQR on marker-relative pose, or polar-coordinate posture controller) to a final pose with stated tolerances (e.g. < 2 cm, < 3°).

- Local reactive safety layer from LiDAR (slow/stop if an obstacle is within a threshold).

- All inputs come from the EKF estimate. Report tracking error RMS, max error, time to dock.

## Part E — Integration and robustness

- A top-level state machine: localise → plan → track → acquire marker → dock → done, with recovery (e.g. rotate to search if the marker is lost).

- Single entry script that runs a full trial headless and one that runs with the viewer for the video.

- Ground truth only in an evaluation/logging module, clearly separated in the code (the markers check this).

- Batch trials (e.g. 20–50): randomised start pose, randomised obstacle positions, different noise levels.

- Disturbances: push the robot mid-run (data.xfrc_applied), reduce wheel friction (slip), occlude the marker, add camera latency.

- Metrics: success rate, final docking position/heading error, tracking RMSE, EKF RMSE and NEES, time to dock. Diagnose every failure and trace it to model, sensing, estimator or controller.

# 4. Suggested code structure

| File / folder | Contents |
| --- | --- |
| sim/scene.xml, sim/assets/aruco_*.png | Room, obstacles, robot, charging station with marker |
| src/model.py | Unicycle model, discretisation, Jacobians, controllability/observability checks |
| src/sensors.py | Noisy encoder/LiDAR readers, camera rendering, intrinsics |
| src/vision.py | ArUco detection, solvePnP, frame transforms, range/bearing |
| src/ekf.py | EKF predict/update, gating, NEES/NIS |
| src/planner.py | Occupancy grid from LiDAR, A*, trajectory generation |
| src/controller.py | LQR tracker, docking controller, saturation |
| src/mission.py | State machine tying everything together |
| eval/run_trials.py, eval/metrics.py | Batch trials, ground-truth comparison (only place ground truth is read), plots |
| README.md / docs/ | The GitHub project page |
| requirements.txt | mujoco, numpy, scipy, opencv-contrib-python, matplotlib |

# 5. Suggested team split (3–4 people)

| Member | Owns | Also must understand for the viva |
| --- | --- | --- |
| 1 | Modelling (A) + LQR tracker and docking controller (D) | EKF Q/R, frames |
| 2 | Sensing (B): marker, camera calibration, noise, frames, LiDAR grid | Observability, EKF update |
| 3 | EKF (C) + consistency analysis | Controller gains, planner |
| 4 (or shared) | Planner, state machine, trials and metrics (E), project page and video | Every component end to end |

The viva asks each person about their own part and at least one part they did not lead, so everyone needs to be able to walk through the whole pipeline.

# 6. Order of work / milestones

| Phase | Goal | Done when |
| --- | --- | --- |
| 1 | Repo set up, UR3 files removed, marker + dock added to scene | Marker visible in front_camera render |
| 2 | Model derived; noisy sensors; ArUco range/bearing working | Measurement vs ground-truth error plotted |
| 3 | EKF running with robot driven manually | Estimate tracks truth inside 3σ bounds |
| 4 | Planner + LQR tracker closed on EKF estimate | Robot reaches pre-dock waypoint around obstacles |
| 5 | Docking controller + state machine | Full nominal run completes end to end → demo by Week 12 |
| 6 | Batch trials, disturbances, tuning studies | Tables of success rate and errors |
| 7 | GitHub page, 5-min video, contribution + AI statements; viva prep | Submitted on Canvas |
| Optional | Hardware induction and real TurtleBot3 run | Sim vs hardware comparison on project page |

# 7. Submission checklist

- GitHub project page states Track A, group number and team members

- Model equations, controllability and observability analysis

- Frame diagram

- Estimator and controller design with all parameter values (Q, R, LQR weights, gate thresholds) and why they were chosen

- Quantitative results: error metrics, success rates, plots over repeated trials

- Install and run instructions; code runs from a fresh environment

- Ground truth used only for evaluation, and visibly so in the code

- 5-minute demo video: completes the task, at least one varied/disturbed trial, estimate + uncertainty shown next to the true state

- Individual contribution statement for every member

- Generative AI declaration (tools and purpose — includes the Codex-generated starter and any Claude use)

- Acknowledge tutorial / MuJoCo / Robotics Toolbox code that was adapted

# 8. Viva questions to prepare for

- Which states are observable from which sensors, and what happens when the marker leaves view?

- Where did Q and R come from? What happens if R is ten times smaller?

- How were the LQR weights chosen and what limits them (wheel-speed saturation, noise)?

- Walk the frames from a camera pixel to the quantity the controller acts on.

- Point at a result plot and explain why it looks the way it does.

- What failed during development, how was it diagnosed, and what would you change?